Operating Manual for NCBOOT32.EXE

Purpose

Ncboot32.exe is provided to maintain the following functions.

- BOOT function (Maintain the CNC system file, Backup the SRAM, etc.)

- IPL menu of CNC

- Display of CNC power-on screen

- Display of CNC alarm screen

- Reconnection on the communication failure

- Start up the registered application program using data window library

Installation

When the  installion of the HSSB drivers, Ncboot32.exe is copied onto the System folder of Windows.

Refer to the Readme2.txt for detailed description for the driver installation.

Start up

Ncboot32.exe will start up automatically, and stay residents in the system tray, when the Windows starts up.

(Example: Ncboot32.exe icon is located on center.)

Termination

It is not necessary to terminate Ncboot32.exe. If you want to terminate Ncboot32.exe, right-click the icon on the system tray and click "End" on the pop-up menu.

While the window of Ncboot32.exe is opened, the "End" menu cannot be selected.

Start sequence

You can select the start sequence using the hardware switch on the HSSB board of CNC side.

Position 0 (Maintenance mode)

1. Wait until the connection with CNC is made successfully.

2. Display the BOOT screen.

3. Display the IPL screen.

4. Display the CNC power-on screen,

5. Initialize the work area of the data window library.

6. Invoke the registered application programs.

7. Watch the HSSB error and the CNC system alarm.

Position 1 (Normal mode)

1. Wait until the connection with CNC is made successfully.

2. Initialize the work area of the data window library.

3. Invoke the registered application programs.

4. Watch the HSSB error and the CNC system alarm.

HSSB multi-connection

Ncboot32.exe supports the multi-connection of the HSSB.

Each CNC connected by the HSSB is managed by the unit of  "Node". The BOOT screen, the IPL screen and the system alarm screen are opened for the individual node.

Start up the registered application programs

The application programs which use the data window library cannot work normally before the initialization of work area for that library.

These programs can be invoked and executed after the initialization of work area is completed if you register them before hand.

The registration of the application programs should be done for the individual node.

Explanation of each Ncboot32.exe window

* You have to use Mouse to open the Ncboot32.exe window.

Status window

The status window will be opened by double-clicking the icon on the system tray, or clicking "Open" of the pop-up menu which appears when you right-click the icon.

Node:	Node number

Name:	Node name (Defined  previously at the device manager on  Windows 95 or the HSSB control panel applet on the Windows NT)

Bus:	Hardware status (0:Connection error, 1:Connecting)

Status:	Software status (Hexadecimal)

Bit1:	Hardware switch position (0 or 1)

Bit2:	BOOT has completed

Bit3:	IPL has completed

Bit8:	CNC system alarm occurs

Push the [Setting...] button to open the optional setting window.

Optional setting window

Register the application programs using this window.

[Node] :	 Select node for registering the application programs. The application programs for the selected node are listed in the list box.

[New...]:	Register new application program. If the path string contains the space character, path string will be enclosed by the double-quotation, automatically.

[Remove]:	 Remove the selected line.

[Edit]:	Edit the selected line for the command line arguments.

The string "%d" in the argument will converted to the related node number. If you want to use the "%" character, write it as "%%".

BOOT window

[Board]:	Select the board, if the sub boards exist.

[Setting...]:	Change the file location

Select the memory card on CNC or the folder of PC.

You may change the file location at any time.

Maintain the system data

[Load...]:	The file selection window will open, Select the file to be loaded to CNC.

[Save]:	Save the selected system data to the file.

[Check]:	Verify the selected system data.

[Delete]:	Delete the selected system data.

Backup/Restore the SRAM

[Backup]:	Backup the SRAM to the file.

[Restore]:	Restore the SRAM from the file.

The center window shows the operating status.

The filename is decided according as the SRAM size. So you cannot rename it.

File operation

[Delete]:	Delete the selected file.

[Format]:	Format the memory card. This button is enabled when you select the memory card by [Setting...] button.

[Refresh]:	Refresh the file list. Push this button when you change the memory card or the floppy disk.

IPL window

The contents of this window is different depending on the model of CNC. Operate according to the menu.

CNC system alarm window

This window appears when the CNC system alarm occurs.

HSSB communication error window

This screen appears when the communication error occurs while processing BOOT and IPL due to the power off of CNC or the defect of the cable.

This screen does not appear even if the communication error occurs while executing the application program. Ncboot32.exe attempts reconnecting until the error recovers.