Operating Manual for NCBOOT32.EXE

Purpose

Ncboot32.exe is provided to maintain the following functions.

- BOOT function (Maintain the CNC system data, Backup the SRAM, etc.)

- IPL menu of CNC

- Display of CNC power-on screen

- Display of CNC alarm screen

- Reconnection on the communication failure

- Start up the registered application program using data window library

Installation

When the  installation of the HSSB drivers, Ncboot32.exe is copied onto the System folder of Windows, or System32 folder of Windows NT.

Refer to the Readme2.txt for detailed description for the driver installation.

Start up

Ncboot32.exe will start up automatically, and stay residents in the system tray, when the Windows starts up.

(Example: Ncboot32.exe icon is located on center.)

Termination

It is not necessary to terminate Ncboot32.exe. If you want to terminate Ncboot32.exe, right-click the icon on the system tray and click "End" on the pop-up menu.

While the window of Ncboot32.exe is opened, the "End" menu cannot be selected.

Start sequence

You can select the start sequence using the rotary switch on the HSSB board of CNC side. In case of i-series and I.T. type 2, this rotary switch is located on the main PCB.

Position 0 (Maintenance mode)

1. Wait until the connection with CNC is made successfully.

2. Display the BOOT screen.

3. Display the IPL screen.

4. Display the CNC power-on screen,

5. Initialize the work area of the data window library.

6. Invoke the registered application programs.

7. Watch the HSSB error and the CNC system alarm.

Position 1 (Normal mode)	(The Power Mate does not support this mode.)

1. Wait until the connection with CNC is made successfully.

2. Initialize the work area of the data window library.

3. Invoke the registered application programs.

4. Watch the HSSB error and the CNC system alarm.

Position 2 (Asynchronous mode)		(Position 1 for the Power Mate.)

1. CNC starts without waiting for the connection.

2. After the connection has made, PC execute the below mentioned initialization.

3. Initialize the work area of the data window library.

4. Invoke the registered application programs.

5. Watch the HSSB error and the CNC system alarm.

HSSB multi-connection

Ncboot32.exe supports the multi-connection of the HSSB.

Each CNC connected by the HSSB is managed by the unit of  "Node". The BOOT screen, the IPL screen and the system alarm screen are opened for the individual node.

Start up the registered application programs

The application programs which use the data window library cannot work normally before the initialization of work area for that library.

These programs can be invoked and executed after the initialization of work area is completed if you register them beforehand.

The registration of the application programs should be done for the individual node.

Explanation of each Ncboot32.exe window

* You have to use Mouse to open the Ncboot32.exe window.

Status window

The status window will be opened by double-clicking the icon on the system tray, or clicking "Open" of the pop-up menu which appears when you right-click the icon.

Node:	Node number

Name:	Node name (Defined  previously at the device manager on  Windows 95 or the HSSB control panel applet on the Windows NT)

Bus:	Hardware status (0:Connection error, 1:Connecting)

Status:	Software status (Hexadecimal)

Bit1:	Hardware switch position 1

Bit2:	BOOT has completed

Bit3:	IPL has completed

Bit4:	Hardware switch position 2

Bit8:	CNC system alarm occurs

Pop up this window on communication error:

If this box was checked, this window will pop up on communication error.

Push the [Close] button to close this window.

Push the [Setting...] button to open the optional setting window.

Push the [About...] button to open the version information window.

Optional setting window

Register the application programs using this window.

[Node] :	 Select node for registering the application programs. The application programs for the selected node are listed in the list box.

[New...]:	Register new application program. If the path string contains the space character, path string will be enclosed by the double-quotation, automatically.

[Remove]:	 Remove the selected line.

[Edit]:	Edit the selected line for the command line arguments.

The string "%d" in the argument will converted to the related node number. If you want to use the "%" character, write it as "%%".

Example)	When you want to invoke the Basic Operation Package 1 for the node after the initialization of the work area for the Data window library, edit the command line as follows.

"C:\Program Files\Basic Operation Package 1\WinBOP32.exe" /Node=%d

BOOT window

[Board]:	Select the board from this list, if the sub boards exist.

[Setting...]:	Change the file location

Select the memory card on CNC or the folder of PC.

You may change the file location at any time.

Maintain the system data

You can handle the NC system data (Control software, Ladder program, etc.) on this window.

[Load...]:	The file selection window will open, Select the file to be loaded to CNC.

[Save]:	Save the selected NC system data to the file.

[Check]:	Verify the selected NC system data.

[Delete]:	Delete the selected NC system data.

Backup/Restore the SRAM

You can backup and restore the SRAM data on CNC on this window.

[Backup]:	Backup the SRAM to the file.

[Restore]:	Restore the SRAM from the file.

The center window shows the operating status.

The filename is decided according as the SRAM size. So you cannot rename it.

File operation

You can handle the files on the memory card on CNC side or at the PC folder, on this window.

[Delete]:	Delete the selected file.

[Format]:	Format the memory card. This button is enabled when you select the memory card by [Setting...] button.

[Refresh]:	Refresh the file list. Push this button when you change the memory card or the floppy disk.

IPL window

The contents of this window is different depending on the model of CNC. Operate according to the menu.

CNC system alarm window

This window appears when the CNC system alarm occurs.

History

Version 2.05

- The communication error window has been abolished.  Instead of that, the status window will pop up on communication error.  (It is possible to disable to pop up the status window by the check box on the status window.)

- Power Mate-H and Power Mate-D have been supported.

- MDI key driver has been supported.

Version 2.01 to 2.04

- Not shipped.

Version 2.00

- First release.