November 14, 1997

FANUC LTD.

Version 1.0

Installation of drivers for Windows NT 4.0 (English version)

Only Windows NT version 4.0 is installed in this hard disk when it is shipped from FANUC LTD. Please install display driver, touch panel driver, HSSB driver, and/or MDI key driver for yourself, according to your system.

This document shows the procedures for installing drivers for Windows NT 4.0 (English version).

[1] Outline

This document describes the procedures to install the following drivers onto the i Series (with PC function) or Intelligent Terminal Type 2 (hereafter, “Device”) in which Windows NT version 4.0 (English version) has been installed in advance.

1.  Display driver

2.  Touch panel driver

3.  HSSB driver

4.  MDI key driver

Note) You must install these drivers in the above order (except drivers which are not needed to your system). For the Device which has both touch panel and MDI key, you must install touch panel driver first, and then MDI key driver. Otherwise, you will need the restoration setup by Windows NT Setup Disk.

To install these drivers, you must have the Administrator privilege. Please logon to Windows NT as the Administrator.

You can use SCSI CD-ROM drive for your device with using SCSI card on the PCMCIA slot. Section 7 shows an example of the method to use CD-ROM drive.

[2] Things to be prepared

* Open CNC Drivers Disk (A02B-0207-K791#ZZ07 Edition 1.1 or later)

* Floppy drive

* Full keyboard

* Mouse

[3] Installation of display driver

I  Driver Setup

1.   Logon to Windows NT as the Administrator.

2.   Open Control Panel.

Click the “Start” button and point to “Settings”. Then click “Control Panel”.

3.   Double-click “Display” icon.

4.   Open “Settings” tab and click “Display Type...” button.

5.   Click “Change...” button.

6.   Click “Have Disk...” button.

7.   Insert “A02B-0207-K791#ZZ07-1” into floppy drive and enter “A:\DISP\NT40” onto “Copy manufacturer’s files from:”. Then click “OK” button.

8.   Select “NeoMagic MagicGraph 128/V/ZV” and click “OK” button.

9.   Click “Yes” button at “Third-party Drivers.” dialogue window.

10.  When the following message is displayed on the screen, click “OK” button.

“The drivers were successfully installed.”

11.  Click “Close” button at “Display Type” dialogue window and “Display Properties” dialogue window.

12.  Remove the floppy disk from floppy drive. When the following message is displayed on the screen, click “Yes” button.

“Do you want to restart your computer now?”

II  Display Settings

13.  When Windows NT comes up, “Invalid Display Settings” dialogue window is displayed on the screen. Click “OK” button.

14.  Set the following values on the items at “Display Properties” dialogue window.

Color Palette           :  256 Colors

Desktop Area          :  640 by 480 pixels

Refresh Frequency  :  60 Hertz

Then click “OK” button.

[4] Installation of touch panel driver

When your Device has touch panel, do the following steps to install the driver.

I  Driver Setup

1.   Logon to Windows NT as the Administrator.

2.   Open Control Panel.

3.   Double-click “Mouse” icon.

4.   Open “General” tab and click “Change...” button.

5.   Click “Have Disk...” at “Select Device” dialogue window.

6.   Insert “A02B-0207-K791#ZZ07-1” into floppy drive. Enter “A:\TOUCH\NT40” onto “Copy manufacturer’s files from:” and click “OK” button.

7.   “Fujitsu Takamisawa component Fidmouse” will be displayed at “Models:”. Click “OK” button.

8.   Click “Close” button at “Mouse Properties” dialogue window.

9.   Remove the floppy disk from floppy drive. When the following message is displayed on the screen, click “Yes” button.

“Do you want to restart your computer now?”

II  Execution of Calibration program

10.  When Windows NT comes up, insert “A02B-0207-K791#ZZ07-1” into floppy drive.

11.  Open Windows NT Explorer.

Click the “Start” button and point to “Programs”. Then click “Windows NT Explorer”.

12.  Click “3.5 Floppy (A:)” at “All Folders” and double-click “Touch” folder at “Contents of ‘3.5 Floppy (A:)’”.

Double-click “Nt40” folder at “Contents of ‘Touch’”.

13.  Execute Calwinfe (Calwinfe.exe) by double-clicking its file name at “Contents of ‘Nt40’”.

14.  Calibration screen will appear. (The cross hairs in red is displayed on the upper-left corner of the screen.)

15.  Touch the center of the cross hairs with touch-pen. After touching, next cross hairs will appear. Repeat touching 9 times.

16.  After touching all 9 cross hairs, press Enter key.

17.  Again press Enter key to finish this calibration.

18.  Click “(C:)” at “All Folders” on “Windows NT Explorer”. Then point to “File” menu and select “New” and click “Folder”. “New Folder” will come up. Enter “MMC” from full keyboard, then press Enter key.

19.  Copy Calwinfe (Calwinfe.exe) from “\Touch\Nt40” folder of “3.5 Floppy (A:)” into “C:\MMC” folder by dragging it using mouse.

Execute Calwinfe (Calwinfe.exe) under “C:\MMC” folder when you calibrate the touch panel next time.

[5] Installation of HSSB driver

i Series (with PC function) and Intelligent Terminal Type 2 (hereafter, “Device”) have High Speed Serial Bus (HSSB) Type 2 interface in them.

The Device without ISA extension unit uses I/O port address 280H - 28FH as HSSB interface. You can’t change this address.

In case of the Device with ISA extension unit, you can select the I/O port addresses by the rotary switches. In case of HSSB multi-connection, determine the I/O port addresses not to conflict with the other HSSB interface boards and the other computer resources, before doing the following steps.

Each device which has HSSB interface is given “Node” number and distinguished by node numbers in Windows. This is true for the device which has only one HSSB interface (its node number equals 0).

I  Driver Setup

1.   Logon to Windows NT as the Administrator.

2.   Close all application programs. You must close the Control Panel if it is running.

3.   Open Windows NT Explorer.

4.   Insert “A02B-0207-K791#ZZ07-2” into floppy drive.

5.   Click “3.5 Floppy (A:)” at “All Folders” and double-click “Hssb” at “Contents of ‘3.5 Floppy (A:)’”.

Then double-click “Nt40” at “Contents of ‘Hssb’”.

6.   Right-click hssb (hssb.inf) file at “Contents of ‘Nt40’” and click “Install” on the pop up menu. Installer will copy driver files and register them to the registry.

II  HSSB (Node) Settings

7.   Open Control Panel.

8.   Double-click “HSSB” icon.

9.   Select the node number corresponding to the HSSB interface and click “Setting...” button.

10.  Enter proper name which indicates the machine connected to the HSSB interface (for example, “FS160 - No.1”, “Milling Machine”) to “Node Name:” within 19 characters.

11.  Select CNC connected to the HSSB interface from the “CNC Type:” list box.

12.  In case of HSSB multi-connection, select I/O port address for the HSSB interface from the “I/O Port Address:” list box.

13.  Click “OK” button to close this window.

14.  In case of HSSB multi-connection, go back to step 9. and repeat these steps for each connected HSSB and CNC.

If a node is not attached to the HSSB interface, select “Disable” from the “I/O Port Address:” list box. After driver installation, I/O port address is set to 280 - 28F for node-0, “Disable” for other nodes as default value.

15.  Click “Close” button.

A dialogue window which asks you to restart Windows will open.

16.  Remove the floppy disk from floppy drive. Click “Yes” button to restart Windows NT.

[6] Installation of MDI key driver

When your Device has MDI key board, do the following steps to install the driver.

I  Driver Setup

1.   Logon to Windows NT as the Administrator.

2.   Close all application programs. You must close the Control Panel if it is running.

3.   Open Windows NT Explorer.

4.   Insert “A02B-0207-K791#ZZ07-2” into floppy drive.

5.   Click “3.5 Floppy (A:)” at “All Folders” and double-click “Mdi” at “Contents of ‘3.5 Floppy (A:)’”.

Then double-click “Nt40” at “Contents of ‘Mdi’”.

6.   Right-click mdi (mdi.inf) file at “Contents of ‘Nt40’” and click “Install” on the pop up menu. Installer will copy driver files and register them to the registry.

7.   “System Settings Change” dialogue window appears on the screen. When the following message is displayed on the dialogue, remove the floppy disk from floppy drive and click “Yes” button.

“Do you want to restart your computer now?”

Please refer to “Readme3.txt” in “MDI” folder to change the MDI keyboard settings.

It is not possible to input from MDI keyboard in the screen before start of Windows, such as the Boot selection screen. So, we recommend you to shorten the waiting time at the Boot selection screen.

1.  Open Control Panel.

2.  Double-click “System” icon.

3.  Click “Startup/Shutdown” tab.

4.  Set the Boot selection time for about 5 seconds on the “Show list for” in the “System Startup” area.

5.  Click “OK” button.

[7] How to use CD-ROM drive

You can use SCSI CD-ROM drive for your Device through the SCSI card connected to the PCMCIA card slot. The following example describes the method to use CD-ROM drive by adopting “Adaptec APA-1460A SlimSCSI” as the SCSI card.

I  Install

1.   Shut down Windows NT and turn off the power of the Device. Then connect APA-1460A to the PCMCIA card slot and CD-ROM drive.

2.   Turn on the Device after turning on the CD-ROM drive.

3.   Logon to Windows NT as the Administrator.

4.   Open Control Panel.

5.   Double-click “PC Card (PCMCIA)” icon.

6.   “New SCSI PC Card Device Found” dialogue window will appear. Click “OK” button.

7.   The following message is displayed on “Insert Disk” dialogue window.

“Please insert the floppy disk labeled ‘Windows NT Workstation CD-ROM’ into drive D: and then click OK.”

Insert “Windows NT Workstation Version 4.0 Setup Disk 3” into floppy drive, then click “OK” button.

8.   Click “Browse...” button at “Files Needed” dialogue window.

Select “Sparrow.sys” in 3.5 Floppy (A:) and click “Open” button at “Locate File” dialogue window.

Click “OK” button at “Files Needed” dialogue window.

9.   “System Settings Change” dialogue window appears on the screen. When the following message is displayed on the dialogue, remove the floppy disk from floppy drive and click “Yes” button.

“Do you want to restart your computer now?”

II  Removing SCSI card

After removing the SCSI card from your Device, an error message will appear at the next logon to Windows NT. So you should do the following steps to delete the SCSI driver when the CD-ROM drive is no more needed. If you want to use CD-ROM drive again after deleting the SCSI driver, please follow the preceding steps “I  Install”.

1.   After turning off the power of the device, remove the SCSI card from the PCMCIA card slot of the Device.

2.   Turn on the Device and logon to Windows NT as the Administrator.

3.   When Windows NT comes up, an error message will appear on “Service Control Manager” dialogue window. Click “OK” button.

4.   Open Control Panel.

5.   Double-click “SCSI Adapters” icon.

6.   Click “Drivers” tab at “SCSI Adapters” dialogue window. Select “Adaptec SlimSCSI 16-Bit PCMCIA SCSI Host Adapter  (Not Started)” and click “Remove” button.

7.   When the following message is displayed on the screen, click “Yes” button.

“Are you sure you want to remove this driver?”

8.   Click “OK” button at “SCSI Adapters” dialogue window. Close Control Panel.