##### FANUC OPEN CNC

FISLIB Standard Library Function

Operator’s Manual

###### INTRODUCTION

This manual describes the information necessary for developing the application software of the FANUC Series 160is/180is/210is.

###### COPYRIGHT, OTHERS

* The copyright of other software provided with FANUC Series 160is/180is/

210is is owned by Microsoft Corporation(U.S.A.) and FANUC Limited.

* No part of the software described above, or its manuals, may be used or

reproduced without permission.

* WindowsCE is registered trademark of Microsoft corporation, U.S.A.

* Visual C++ is trademark of Microsoft corporation, U.S.A.

* WindowsCE Toolkit for Visual C++ is trademark of Microsoft corporation, U.S.A.

* Company name and product name mentioned in this manual are registered trademark or

trademark of each company.

- Outline

FISLIB is a library to register files to database or edit the database in order to perform saving files at power off and  restoring files or invoking application programs at power on. The database  is possible to be edit on NCBOOTis status window.

- Library files

FISLIB.DLL	: The DLL library

FISLIB.LIB	: The import library

FISLIB.H	: The header file for prototype of function, structure and definition of return status

( for Microsoft Visual C++ )

FISLIB.BAS	: The header file for prototype of function, structure and definition of return status

FISLIB.DOC	:  FISLIB standard library function operator’s manual  (this file)

- Library functions

[Functions to register a file or delete a file registry and get information of a file registry]

Function			Name and parameters

Register restored file		WORD FISLIB_Add_Load(

LPCTSTR cFileName, WORD number, DWORD size );

Get information about		BOOL FISLIB_Get_Load(

restored file		LPCTSTR cFileName, WORD number, DWORD size );

Delete registry of restored file	BOOL FISLIB_Del_Load(

LPCTSTR cFileName );

Register saved file		WORD FISLIB_Add_Save(

LPCTSTR cFileName, WORD number, DWORD size );

Get information about		BOOL FISLIB_Get_Save(

saved file		LPCTSTR cFileName, WORD number, DWORD size );

Delete registry of saved file	BOOL FISLIB_Del_Save(

LPCTSTR cFileName );

Register invoked file		WORD FISLIB_Add_Start(

LPCTSTR cFileName, WORD number );

Get information about		BOOL FISLIB_Get_Start(

invoked file		LPCTSTR cFileName, WORD number );

Delete registry of invoked file	BOOL FISLIB_Del_Start(

LPCTSTR cFileName );

Get result status of last save 	BOOL FISLIB_Check_Save(

execution		LPCTSTR cFileName );

Get result status of last restore	BOOL FISLIB_Check_Load(

execution		LPCTSTR cFileName );

* Specify file name to cFileName as follows.

- A file in the object store

Specify “OBJ:file path  (OBJ is possible to be omitted)”

example)  \temp\workfile.dat

- A memory mapped file

Specify “MMF:ID name”

example) MMF:App_Temp_Data

* Specify registry order number(1, 2, 3, …) to the “number” parameter. Specify zero in case addition.

If the result status number is zero, it mean that the function failed.

*Specify data length of memory mapped file to “size” parameter.  “size” parameter is not effective

for object store file.

[Function to register process]

If an application has been registered own window handle to NCBOOTis process table, NCBOOTis send WM_DESTORY massage to the application at power-off.  So the application can execute own shut down process. The handle is possible to get by any application, so the handle is useful for various purposes.

Function			Name and parameters

Register handle		BOOL FISLIB_Add_Proc(

HWND hWnd, WORD ProcID );

Delete registory		BOOL FISLIB_Del_Proc(

WORD ProcID );

Get handle			HWND FISLIB_Get_Proc(

WORD ProcID );

* Specify 64 or larger number and below 128 to ProcID parameter.

Define the number as follows:

#define FISLIB_PROC_Apl1	FISLIB_PROC_USER +0

#define FISLIB_PROC_Apl2	FISLIB_PROC_USER +1

Refer fislib.h.

#define FISLIB_PROC_NCBOOTis	0	// NCBOOTis

#define FISLIB_PROC_CNCScrn	1	// CNCScreen Display Function

#define FISLIB_PROC_USER		64	// Top No. for user application

#define FISLIB_PROC_MAX		127	// Last No. for user application

* If the function to get handle returned NULL value as window handle, the process is inactive.