##### FANUC OPEN CNC

NCBOOTis

Operator’s Manual

###### INTRODUCTION

This manual describes the information necessary for operating the FANUC Series 160is/180is/210is.

###### COPYRIGHT, OTHERS

* The copyright of other software provided with FANUC Series 160is/180is/210is is owned by

Microsoft Corporation(U.S.A.) and FANUC Limited.

* No part of the software described above, or its manuals, may be used or  reproduced without

permission.

* WindowsCE is registered trademark of Microsoft corporation, U.S.A.

* Company name and product name mentioned in this manual are registered trademark or

trademark of each company.

- Purpose

NCBOOTis (Ncbootis.exe) is provided to maintain the following functions.

- BOOT function (Install the CNC system data, Backup the SRAM, etc.)

- IPL menu of CNC (Maintain the CNC system data: Clear SRAM, etc.)

- Display of CNC power-on screen

- Display of CNC alarm screen

- Reconnection on the communication failure

- Adjust the calendar and clock

- Register or delete the files which are saved, restored and started automatically

- Save the registered files at power-off

- Restore the registered files at power-on

- Start up the registered application program at power-on

- Start up

NCBOOTis will start up automatically, and stay residents in the system tray, when the Windows CE starts up.

(Example: NCBOOTis icon is located on left.)

- Termination

NCBOOTis can not be terminated.

- Start sequence

You can select the start sequence using the rotary switch on the main board of CNC.

- Position 0 (Maintenance mode)

1. Wait until the connection with CNC is made successfully.

2. Display the BOOT screen.

3. Display the IPL screen.

4. Display the CNC power-on screen.

5. Wait until the battery charge complete.

6. Restore the registered files.

7. Invoke the registered application programs.

8. Watch the communication error and the CNC system alarm.

- Position 1 (Normal mode)

1. Wait until the connection with CNC is made successfully.

2. Wait until the battery charge is completed.

3. Restore the registered files.

4. Invoke the registered application programs.

5. Watch the communication error and the CNC system alarm.

- Position 2 (Asynchronous mode)

1. CNC starts without waiting for the connection.

2. After the connection has made, PC executes the below mentioned initialization.

3. Wait until the battery charge is completed.

4. Restore the registered files.

5. Invoke the registered application programs.

6.Watch the HSSB error and the CNC system alarm.

- Position 7 (Switching Maintenance mode/Asynchronous mode)

it is same as  position 0 (Maintenance mode) in the case "6" and "7" keys of MDI is pressed when CNC power is turned on.

it is same as  position 1 in other cases.

- Functions detail

- Save and restore files automatically

The files in the object store (DRAM file system) will be erased when the system shut down. If the files have been registered to NCBOOTis database, the files are saved at system shut down, and then the files are restored at the system start up. The available total quantity of files is less than 1MB.

- Start up the registered application programs

The application programs which have been registered in NCBOOTis can be invoked and executed after the initialization of work area is completed.

- Adjust calendar and clock

NCBOOTis set up the calendar and clock by referring the RTC (Real Time Clock) on CNC at power-on. Adjustment of the calendar and clock and time zone is possible.

- Explanation of each NCBOOTis window (The examples of WindowsCE2.12 version)

- Status window

The status window will be opened by double tapping the icon on the system tray, or by restarting the NCBOOTis.

Edit database for saving and restoring files and invoking application programs.

- Push the [Close] button to close this window.

- Push the [About...] button to open the version information window.

- Select the CNC type in [CNC & keyboard type] box.

- Select the data type for saving or restoring or invoking files in [List] box.

- Push the [Insert] button to register new file.

- Push the [Delete] button to delete registry of file.

- Push the [Up] or [Down] button to change registered order.

- Push the [Clock] button to adjust calendar and clock.

- If [Screen saver enable] box was checked, the screen saver function will be available.

- If the [Auto start NCBOOTis] box was checked, NCBOOTis will be invoked automatically at

power on. When you want to update the Ncbootis.exe, make this box  unchecked and restart

the CNC.

- If [Backup unit enable] box was checked, the battery will be used to save the files at power

off.  If this box was not checked, push the [Save] button to save files.

- If the [High priority] box was checked, the priority of thread to save files is set highest.

- If the [Alarm for backup] box was checked, the warning message will be appear

when specified days passed until power on.

- BOOT window

- [Setting...] : Change the file location (folder).

- [Close] : Push this button to close this window.

- Maintain the system data

You can handle the NC system data (Control software, Ladder program, etc.) on this window.

- [Load...] : The file selection window will open, Select the file to be loaded to CNC.

- [Save] : Save the selected NC system data to the file.

- [Check] : Verify the selected NC system data.

- [Delete] : Delete the selected NC system data.

- Backup/Restore the SRAM

You can backup and restore the SRAM data on CNC on this window.

- [Backup]:	Backup the SRAM to the file.

- [Restore]:	Restore the SRAM from the file.

- The center window shows the operating status.

- The filename is decided according as the SRAM size. So you cannot rename it.

- File operation

You can handle the files in the ATA card and object store, on this window.

- [Delete]:	Delete the selected file.

- [Refresh]:	Refresh the file list. Push this button when you change the memory card.

- IPL window

The content of this window is different depending on the model of CNC. Operate according to the menu.

- CNC system alarm window

This window appears when the CNC system alarm occurs.